# METHOD OF STATEMENT

Date: 29th Feb 2024

Description of work

To hoist down 1 case from forwarder lorry chassis to the ground, uncrate, shift and position into production area at ground floor using pallet truck

Machine Spec

1 unit Die Cut machine

Case Dim: 290 x 90 x 240 cm

Case Wt: 2, 350 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

06-08/03/24

Time of Operation

10:00 am

Location of Operation

#### 10 Tuas South St 5

CDI Energy Products Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Shackle

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from forwarder lorry to the ground

- Uncrate case and put aside

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing crane to unloading area

- Insert pallet truck under machine base

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175