# METHOD OF STATEMENT

Date: 19th June 2020

Description of work

To hoist down 1 unit sheltet from lorry chassis to the ground, insert rollers, shift and position to designated location at ground floor

Sheltet Spec

Dim: 400 x 240 x 230 cm

Wt: 5, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/06/2020

Time of Operation

Est: 12:00 pm

Location of Operation

#### 100 Jurong East St 21

ST Electronics

Equipment Use

1 x 633 lorry loader – PALFINGER PK92002SHH

Rollers/pallet truck

1 set wire sling

1 set shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Attached lifting gears to crane hook and secure onto hoisting point of load to be lifted

- Hoist up 1ft above ground to check on balancing

- When well balance, hoist down sheltet from lorry chassis

- Insert rollers under sheltet base

- Transport and position sheltet at the designated location

- Unload sheltet from rollers and let it sit onto 8” x 1ft woodblock

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175