# METHOD OF STATEMENT

Date: 20th Dec 2019

Description of work

To unload and hoist down 2 unit new machine from M’sia lorry, shift and position into production area

Machine Spec

2 unit Brother TC31A

Wt: 2, 000 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/12/2019

Time of Operation

09:45 am

Location of Operation

#### 10A Joo Koon Circle

SPEQS Manufacturing Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling onto machine hoisting point

- Hoist down machine from M’sia lorry chassis to near ground level

- Swing crane to near door entrance

- Insert pallet truck under machine base

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175