# METHOD OF STATEMENT

Date: 20th Aug 2020

Description of work

To unload and hoist down 1 unit equipment from lorry chassis, load onto table, shift and position into production at ground floor

Equipment Spec

Dim: 2122 x 270 x 427 mm

Wt: 152 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

25/08/2020   -   11:00 am

Location of Operation

#### 11 Calshot Rd

SAESL

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling onto equipment hoisting point

- Hoist down equipment from lorry chassis and load onto table

- Insert pallet truck

- Shift and transport equipment into production at ground floor

- Position equipment at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175