# METHOD OF STATEMENT

Date: 18th Dec 2023

Description of work

To unload and hoist down 3 unit equipment from lorry chassis, shift and position into customer premise at ground floor

Equipment Spec

Dim: 216 x 140 x 140 cm each

Wt: 550 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

21/12/23

Time of Operation

11:40 am

Location of Operation

#### 11 Stockport Rd

DSO Marina Hill

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert sling onto crane hook

- Secured sling to rigging point of equipment

- Hoist up equipment from lorry chassis

- Swing crane and lower down equipment to near ground level

- Insert pallet truck under equipment base

- Shift and transport into factory premise at ground floor

- Position equipment at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175