# METHOD OF STATEMENT

Date: 19th Mar 2020

Description of work

To unload and hoist down 1 unit machine and accessories from low-loader/lorry chassis, shift and position into production area at ground floor

Machine Spec

1 unit Power Master

Dim: 536 x 325 x 277 cm

Wt: 11, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 149 Gul Circle

FMC Technologies (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x low loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Metal plate (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Attached one end of sling to crane hook and secured another end onto load to be lifted

- Hoist up machine about 1ft above ground to check on balancing

- When well balanced, hoist down machine from low loader/lorry chassis to the ground

- Swing crane to near door entrance

- Lower down machine to near ground level

- Insert rollers under machine bed

- Transport machine into production area

- Position machine at the designated location

- Unload machine from rollers

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175