# METHOD OF STATEMENT

Date: 26th Dec 2019

Description of work

To unload 1 unit machine from forwarder lorry, shift and position into production area at ground floor

Machine Spec

1 unit FH480 Conventional Lathe

Dim: 280 x 120

Wt: 1, 650 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/12/2019

Time of Operation

10:00 am

Location of Operation

#### 149 Gul Circle

FMC Technologies (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Unload machine from forwarder lorry to the ground

- Hoist up and detach machine from base

- Swing crane to near door entrance

- Insert rollers under machine

- Transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175