# METHOD OF STATEMENT

Date: 06th Feb 2020

Description of work

To unload and hoist down 2 unit Trolley and 2 unit Base Plate from lorry chassis, shift and position into premise at ground floor

Trolley Spec

Dim: 220 x 100 x 100 cm

Est wt: 3-4 ton each

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

07/02/2020   -   11:30 am

Location of Operation

#### 158A Gul Circle

DNV GL Laboratory

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling onto trolley hoisting point

- Hoist down trolley from lorry chassis to near ground level

- Insert pallet truck under trolley base

- Shift and transport into production

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175