# METHOD OF STATEMENT

Date: 24th May 2021

Description of work

To hoist down and unload 1 unit machine from lorry chassis, insert pallet truck to machine bed, shift and position into production area at ground floor

Machine Spec

1 unit Troop CHMER CMA64C

Dim: 235 x 221 x 234 cm

Wt: 3, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 28/05/21

Time of Operation

09:45 am

Location of Operation

#### 2 Pioneer Sector 3

GE Repair Solutions (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

1 set shackle

Canvas/metal sheet (for floor protection)

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate case

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine base

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175