# METHOD OF STATEMENT

Date: 15th May 2019

Description of work

Using lorry crane to unload and hoist down 1 unit machine from lorry chassis, insert rollers to machine base, shift and position into production area

Machine Spec

1 unit VMX 42SRTi CNC Milling

Dim: 254 x 222 x 260 cm

Wt: 8, 400 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/06/2019

Time of Operation

12:00 pm

Location of Operation

#### 2 Pioneer Sector 3

GE Repair Solutions (S) Pte Ltd

Equipment Use

1 x 633 lorry loader - PALFINGER 100002F

1 x forklift

Rollers/pallet truck

1 set wire sling

1 set shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift driver will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers under machine base

- Using forklift to assist in pulling/pushing machine

- Transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175