# METHOD OF STATEMENT

Date: 12th Mar 2024

Description of work

To unload and hoist down 4 packages of 1 unit machine and accessories from lorry chassis to the ground, insert rollers/pallet truck to machine base, shift and position into factory workshop area at ground floor

Machine Spec

Hydraulic Press Brake PPED-5 135/30 machine

Dim: 4112 x 1847 x 2815 mm / Wt: 7, 800 kg

Dim: 1200 x 800 x 750 mm / Wt: 140 kg

Dim: 1200 x 800 x 740 mm / Wt: 308 kg

Dim: 1200 x 800 x 540 mm / Wt: 234 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/03/24

Time of Operation

10:30 am

Location of Operation

#### 25 Airline Rd, Airline House

Structural Workshop Core 01E

Equipment Use

2-3 x 633 lorry loader

Rollers/pallet truck

Wire sling

Webbing sling

Shackle

Hydraulic/hand jack

Woodblock

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist down machine from lorry chassis to near ground level

- Swing crane to unloading area

- Insert rollers under machine base

- Transport machine into workshop area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175