# METHOD OF STATEMENT

Date: 28th Jan 2021

Description of work

To unload and hoist down 1 unit machine and accessories from low-loader/lorry chassis, shift and position into production area at ground floor

Machine Spec

1 unit Max-200 Patex Press machine

Dim: 280 x 230 x 380 cm

Wt: 40, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

28/01/2021

Time of Operation

10:00 am

Location of Operation

#### 28 Tuas Ave 2

Equipment Use

1 x 160 ton mobile crane

1 x 633 lorry loader

Low-bed

Rollers

Wire sling

Shackle

1 x forklift

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane lorry at designated unloading area

- Reverse low-bed to near crane

- Attached required sling to crane hook and secured to hoisting point of machine be lifted

- Hoist up machine about 1ft above low-bed chassis

- Shift out low-bed away

- Lower down machine to the ground

- Using 2 crane to lift up machine to vertical position

- Insert rollers under machine bed

- Transport machine to inside factory premise

- Use of forklift to assist in pulling/pushing the machine

- Position machine at the designated location

- Unload machine from rollers

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175