# METHOD OF STATEMENT

Date: 22nd Nov 2019

Description of work

To unload 3 cases of 1 unit machine from 1 x 20ft GP container, uncrate, shift and position into production area

Machine Spec

XO55-500 3D Coordinate CMM

Case 1: 158 x 148 x 177 cm / Wt: 1, 278 kg

Case 2: 107 x 128 x 130 cm / Wt: 210 kg

Case 3: 140 x 110 x 103 cm / Wt: 235 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/11/2019

Time of Operation

09:30 am

Location of Operation

#### 39 Changi North Crescent

Goodrich Aerospace

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location (back to back with container)

- Pull out case from container onto lorry chassis

- Hoist down case from lorry chassis to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert pallet truck under machine stand

- Transport machine into production area

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175