# METHOD OF STATEMENT

Date: 14th Oct 2019

Description of work

To hoist down 1 unit Test Bench from lorry chassis, shift and position into customer premise at ground floor

Test Bench Spec

Dim: 50 x 50 x 170 cm

Wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/10/2019

Time of Operation

11:30 am

Location of Operation

#### 39 Changi North Crescent

Goodrich Aerospace

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down Test Bench from lorry chassis

- Lower down Test Bench to near ground level

- Insert pallet truck

- Transport into production area at ground floor

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175