# METHOD OF STATEMENT

Date: 09th June 2021

Description of work

To unload 4 cases of 1 unit machine from lorry chassis to the ground, uncrate, insert rollers to machine base, shift and position into production area at ground floor

Machine Spec

LH 1210-2500 CMM machine

Case 1: 420 x 220 x 137 cm / Wt: 9, 600 kg

Case 2: 228 x 100 x 100 cm / Wt: 403 kg

Case 3: 218 x 195 x 110 cm / Wt: 491 kg

Case 4: 135 x 95 x 160 cm / Wt: 130 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/06/2021

Time of Operation

10:00 am

Location of Operation

#### 40 Lok Yang

Shine Precision

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Shackle

Hand jack

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case to the ground

- Uncrate case

- Insert sling to crane hook and secure to machine hoisting point

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers under machine base

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175