# METHOD OF STATEMENT

Date: 21st July 2020

Description of work

To hoist down 1 unit machine from lorry chassis, shift and position to 4th floor via cargo lift

Machine Spec

1 unit Drill 20

Dim: 134 x 91 x 194 cm

Wt: 870 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 11/08/20   -   11:00 am

Time of Operation

09:45 am

Location of Operation

#### 45 Changi South Ave 2

Cicor Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case

- Insert sling onto machine hoisting point

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine base

- Shift and transport machine to 4th floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175