# METHOD OF STATEMENT

Date: 08th Apr 2019

Description of work

Using lorry crane to unload 1 unit machine from lorry chassis, insert rollers to machine base, shift and position into production area

Machine Spec

1 unit VL-55 CNC Lathe

Dim:

Wt: 12, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/04/2019

Time of Operation

13:00 pm

Location of Operation

#### 51 Corporation Rd

Avitron Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x lorry crane

Pallet truck/rollers

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated unloading area

- Hoist down machine from lorry chassis and lower down slowly to near ground level

- Swing crane to unloading area

- Insert rollers under machine base

- Transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175