# METHOD OF STATEMENT

Date: 20th Jan 2020

Description of work

To unload 2 unit Grinding machine from lorry chassis, shift and position into production area at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/01/2020

Time of Operation

10:00 am

Location of Operation

#### 51 Marsilling Rd

Beyonics International Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Unload machine from forwarder lorry to the ground

- Hoist up and detach machine from base

- Insert pallet truck under machine

- Transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175