# METHOD OF STATEMENT

Date: 14th Apr 2021

Description of work

To hoist down 2 pallet of equipment from lorry chassis, using pallet truck to shift and position into factory premise

Equipment Spec

Horizontal Profile Projector

Dim: 145 x 85 x 146 cm

Wt: 290 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

16/04/21

Time of Operation

10:00 am

Location of Operation

#### 6 Tuas Ave 18

Rohag (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down projector from lorry chassis and load onto table

- Insert pallet truck

- Shift and transport into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175