# METHOD OF STATEMENT

Date: 24th May 2022

Description of work

1) To unload and hoist down 1 unit used machine from lorry chassis, shift and position into production at ground floor

2) To shift out 2 units used machine from 8 Tractor Road load on to lorry chassis, secure with rachet belts.

Machine Spec

2 unit Used Machine

Dim: 100 x 180 x 195 cm

Wt: 150 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/05/22

Time of Operation

9:30 am

Location of Operation

#### 8 Tractor Rd

Excelitas Technologies (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case

- Insert sling onto crane hook and secured to hoisting point of machine

- Hoist down machine from lorry chassis to the ground

- Insert pallet truck under machine base

- Shift and transport machine into production at ground floor

- Position machine at the designated location

- To shift out 2 unit old machine from production to loading area

- Hoist up and load onto lorry chassis

- Secured with strapping belt

- Transport and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175