# METHOD OF STATEMENT

Date: 08th Dec 2020

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, shift and position into production area at ground floor

Machine Spec

1 unit Okamoto LN350B Grinder

Wt: 840 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

10/12/20   -   11:00 am

Location of Operation

#### 9 Sixth Lok Yang Rd

Singapore Kobe Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling onto machine hoisting point

- Hoist down machine from lorry chassis to the ground

- Insert pallet truck under machine base

- Shift and transport machine into production at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175