# METHOD OF STATEMENT

Date: 02nd Dec 2019

Description of work

1) To hoist down 2 unit new machine from lorry chassis, shift and position into production area

2) To shift out 2 unit machine from production to loading area, hoist up onto lorry chassis and deliver to another factory

Machine Spec

Move in

1 unit FH660 Conventional Lathe

1 unit FH480 Conventional Lathe

Move out

1 unit San Shing Lathe

1 unit SK2380 Lathe

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 10/12/2019

Time of Operation

09:40 am

Location of Operation

#### 21 Loyang Crescent

Safran Landing Systems Services (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck/rollers

Wire/webbing sling

Shackle

Metal sheet/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling onto machine hoisting point

- Hoist down machine from lorry chassis to near ground level

- Swing crane to near door entrance

- Insert rollers under machine base

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175