# METHOD OF STATEMENT

Date: 17th Aug 2020

Description of work

1) To hoist down 1 unit machine from lorry chassis, shift and position into production area at ground floor

2) To shift out 1 unit old machine from production to loading area, hoist up onto lorry chassis and deliver to another location

Machine Spec

Move in

1 unit Okamoto ACC63 Grinder

Dim: 2740 x 2120 x 1845 mm / Wt: 2, 800 kg

Move out

1 unit Okamoto ACC52ST Grinder

Dim: 2430 x 1920 x 1845 mm / Wt: 2, 100 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/08/2020

Time of Operation

10:00 am

Location of Operation

#### 27 Senoko South Rd

Heitkamp Und Thumann (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down machine from lorry chassis to near ground level

- Insert pallet truck under machine base

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- To shift out 1 unit old machine from production to loading area

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175