# METHOD OF STATEMENT

Date: 28th Nov 2022

Description of work

To unload 3 skid from 1 x 40ft HQ container, shift and position into production area at ground floor

Spec

3 skid of 1 unit Straightener Feeder

Straightener Feeder dim: 180 x 180 x 197 cm / Wt: 2, 200 kg

Uncoiler dim: 300 x 210 x 249 cm / Wt: 3, 650 kg

Control box dim: 101 x 141 x 150 cm / Wt: 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

01/12/22

Time of Operation

09:45 am

Location of Operation

#### 11 Bulim Walk

Shimano (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers / pallet truck

Hand jack

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry back to back with container

- Crane to pull out skid from container onto lorry chassis

- Hoist down skid to the ground

- Attached required lifting gears to crane hook and secured to rigging point of equipment

- Lorry loader to hoist up and detach equipment from base

- Swing equipment away from base

- Lower down to near ground level

- Insert rollers/pallet truck under equipment bed

- Shift and transport into factory premise area at ground floor

- Position at the designated location

- Unload from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175