# METHOD OF STATEMENT

Date: 24th May 2022

Description of work

To unload 1 case from lorry chassis to the ground, uncrate, using pallet truck to shift and position into factory premise at ground floor

Machine Spec

Move in new machine

1 unit Explorer 08.10.06

Dim: 1823 x 1350 x 2658 mm

Wt: 1, 265 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/07/22

Time of Operation

09:45 am

Location of Operation

#### 23 Loyang Way

GE Aviation

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine onto machine stand

- Insert pallet truck under machine stand

- Shift and transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175