# METHOD OF STATEMENT

Date: 30th Aug 2024

Description of work

To hoist down 1 case of 1 unit machine from lorry chassis to the ground, uncrate, using pallet truck to shift and position into CMM room at ground floor

Machine Spec

1 unit Glory Scan 08.10.06 CMM machine

Case dim: 2938 x 1562 x 2502 mm

Wt: 1, 804 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/09/24

Time of Operation

09:45 am

Location of Operation

#### 1 Pandan Crescent

Emerson

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar/woodblock

Battery drill

To lay out onto flooring for floor protection – 1 layer of canvas, 1 layer of PVC

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis to the ground

- Use of battery operated drill to loosen and remove all the screws on the crate

- After all screw are removed, workers to carefully take off the crate by parts

- Place all parts of crate to one side

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine onto machine stand

- Insert pallet truck under machine stand

- To lay out 1 layer of canvas, 1 layer of onto flooring for floor protection

- Shift and transport machine into CMM room at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175