# METHOD OF STATEMENT

Date: 18th Jan 2024

Description of work

1) To hoist down 1 case of 1 unit machine from lorry chassis to the ground, uncrate, using pallet truck to shift and position into factory premise

2) To shift out 1 used Mitutoyo KN807 CMM to loading area, hoist up onto lorry chassis for disposal

3) To relocate 1 unit 7.10.7 CMM within QA room

New Machine Spec

1 unit 9.12.8 CMM machine

Case dim: 3538 x 1878 x 2439 mm

Wt: 2, 555 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/01/24 (Mon)

Time of Operation

09:45 am

Location of Operation

#### 9 Tuas Dr 1

Fuyu Corporation Ltd

Equipment Use

1-2 x 633 lorry loader

Pallet truck

Wire/webbing sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175