# METHOD OF STATEMENT

Date: 22nd Dec 2025

Description of work

Lorry crane to hoist down “A” Frame structure from lorry chassis, using pallet truck to shift into workshop and assemble

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 2 A.M.K Drive

ITE College Central

Equipment Use

1 x 633 lorry loader

‘A’ frame with chain block

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secured to rigging point of ‘A’ frame structure

- Hoist up structure from lorry chassis

- Swing crane and lower down structure to the ground

- Release all sling from structure

- Insert pallet truck under structure bed

- Shift and transport into workshop

- Set up ‘A’ frame structure at designated location for Mazak engineer usage

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175