# METHOD OF STATEMENT

Date: 10th May 2023

Description of work

To unload and hoist down 3 unit machine from low loader, upright machine, insert rollers to machine base, shift and position into production area  at ground floor

Machine Spec

2 unit Aida 200ton Single Crank Press / Wt: 18, 600 kg

1 unit Aida 110ton Power Press / Wt: 10, 100 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/05/23

Time of Operation

09:45 am

Location of Operation

#### 4 Tuas Ave 13

IEG Oilfield Services Pte Ltd

Equipment Use

1 x 70ton ton mobile crane

1 x 633 lorry loader

2 x low loader

1 x forklift

Rollers/pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated unloading area

- Reverse low loader to near crane

- Attached sling to crane hook and secured onto rigging point of machine

- Hoist up machine from low loader

- Shift out low loader away

- Lower down machine to near ground level

- Upright machine using mobile and lorry crane

- Insert rollers under machine bed

- Shift and transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175