# METHOD OF STATEMENT

Date: 17th Aug 2022

Description of work

To unload and hoist down 1 unit machine from M’sia low bed, upright machine, insert rollers to machine base, shift and position into production area  at ground floor

Machine Spec

1 unit Aida Press NS2-3000 Double Crank

Dim: 3500 x 2978 x 4456 mm

Wt: 45, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/08/22

Time of Operation

09:45 am

Location of Operation

#### 249 Jalan Boon Lay

ST Engineering Land System

Equipment Use

1 x 160ton mobile crane

1 x 633 lorry loader

1 x forklift

Rollers/pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated unloading area

- Reverse M’sia low bed to near crane

- Attached sling to crane hook and secured onto rigging point of machine

- Hoist up machine from M’sia low bed

- Shift low bed away

- Lower down machine to the ground

- Upright machine

- Insert rollers under machine base

- Shift and transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175