# METHOD OF STATEMENT

Date: 27th Oct 2025

Description of work

To hoist down 1 unit machine from lorry chassis, insert rollers to machine bed, shift and position into production area at ground floor

Machine Spec

1 unit Bruderer BSTA 50-95C2

Dim: 250 x 130 x 140 cm

Wt: 8,000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/11/25

Time of Operation

14:00 pm

Location of Operation

#### 31 Senoko South Rd

Equipment Use

1 x 633 lorry loader

Rollers

Wire sling

Hydraulic/hand jack

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Rigger to insert sling to crane hook

- Secure sling to machine rigging point

- Hoist up machine from lorry chassis

- Lower down machine to near ground level

- Swing crane to unloading area

- Insert rollers under machine bed

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175