# METHOD OF STATEMENT

Date: 15th Apr 2024

Description of work

To unload 1 unit machine from 1 x 20ft container, insert rollers to machine base, shift and position into production area at ground floor

Machine Spec

1 unit BSTL 350-88BS

Dim: 270 x 156 x 260 cm

Wt: 6, 100 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/04/24

Time of Operation

10:00 am

Location of Operation

#### 65 Loyang Way

Bruderer Presses (Far East) Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x platform (to lay out for machine pathway)

Rollers

Wire sling

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine from container

- Lower down machine to near ground level

- Swing crane to unloading area

- Insert rollers under machine bed

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175