# METHOD OF STATEMENT

Date: 20th Nov 2023

Description of work

To unload chip compactor, shift and position into production area at ground floor

Spec

1 crate of Chip Compactor

Dim: 265 x 152 x 213 cm / Wt: 2, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27/11/23

Time of Operation

16:30 pm

Location of Operation

#### 11 Bulim Walk

Shimano (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers / pallet truck

Hand jack

Wire/webbing sling

Canvas/metal plate (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Uncrate case

- Attached required lifting gears to crane hook and secured to rigging point of equipment

- Lorry loader to hoist up and detach equipment from base

- Swing crane and lower down to near ground level

- Insert rollers/pallet truck under equipment base

- Shift and transport into factory premise area at ground floor

- Position at the designated location

- Unload from rollers and insert levelling pad under equipment bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175