# METHOD OF STATEMENT

Date: 11th Sep 2023

Description of work

1) To hoist down 1 unit machine from lorry chassis to the ground, using pallet truck to shift and position into factory premise

2) To shift out 1 unit machine from production to loading bay, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

Move in

1 unit Chevalier FSG-1224 Grinder

Dim: 272 x 176 x 223 cm

Wt: 2, 200 kg

Move out to dispose

1 unit used Okamoto Grinder

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

13/09/23

Time of Operation

09:45 am

Location of Operation

#### 9 Sixth Lok Yang Rd

Singapore Kobelco Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case and put aside

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- To shift out I unit used machine to loading area

- Hoist up onto lorry chassis and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175