# METHOD OF STATEMENT

Date: 15th June 2021

Description of work

To unload and hoist down 1 unit machine from lorry chassis, shift and position into production at ground floor

Machine Spec

1 unit Lathe HG-660 X 1200

Dim: 280 x 155 x 198 cm

Wt: 2, 800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

21-25/06/21

Time of Operation

09:45 am

Location of Operation

#### 55 Loyang Dr

Asian Surface Technologies Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling onto machine hoisting point

- Hoist down machine from lorry chassis

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into production at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175