# METHOD OF STATEMENT

Date: 06th Sep 2023

Description of work

1) To unload/hoist down 1 unit machine and accessories from lorry chassis, insert rollers to machine bed, shift and position into production area at ground floor

2) To shift out 1 unit machine and accessories from production to loading area, hoist and load up onto Malaysia lorry

Machine Spec

Move in

1 unit Hydraulic Press Brake

Dim: 3900 x 2000 x 2555 mm

Wt: 7, 785 kg

Move out

1 unit use LVD Press Brake

Wt: 3900 x 2000 x 2555 mm

Wt: 6, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12/09/23

Time of Operation

09:45 am

Location of Operation

#### 13 Kwong Min Rd

Fabristeel

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

Wire sling / Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secure to machine rigging point

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175