# METHOD OF STATEMENT

Date: 16th Jan 2024

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, shift and position into production area at ground floor

Machine Spec

1 unit Vesta 660

Dim: 2450 x 2000 x 2400 mm

Wt: 4, 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/01/24

Time of Operation

Afternoon schedule

Location of Operation

#### 37 Joo Koon Rd

Energy Pipeline Services Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Wire sling

Shackle

Hand jack/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger will insert sling to crane hook

- Secure sling to machine rigging point

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175