# METHOD OF STATEMENT

Date: 04th May 2023

Description of work

To unload 3 packages of 1 unit Chip Conveyor from lorry chassis, uncrate, shift and position into production area at ground floor

Spec

Crate 1: 276 x 133 x 107 cm / Wt: 450 kg

Crate 2: 457 x 94 x 126 cm / Wt: 600 kg

Crate 3: 211 x 72 x 94 cm / Wt: 350 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/05/23

Time of Operation

09:45 am

Location of Operation

#### 70 Joo Koon Circle

Nakano Metal Press

Equipment Use

2 x 633 lorry loader

Wire/webbing sling

Pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, rigger and signalman will be involved in the operation

# JOB SCOPE

- Deploy lorry loader at designated location

- Hoist down crate from lorry chassis to the round

- Uncrate case and put aside

- Hoist up and detach equipment from base

- Insert rollers/pallet truck under equipment bed

- Shift and transport into factory premise area at ground floor

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175