# METHOD OF STATEMENT

Date: 14th Jan 2025

Description of work

1) To hoist down 3 pcs metal plate from lorry chassis to the ground, shift and position into production area at ground floor

2) To relocate 2 unit Mould machine from existing location to new location within factory premise

Spec

Metal plate

2 pcs plate dim: 244cm x 609cm x 12mm each / Wt: 1, 200 kg each

1 pcs plate dim: 212cm x 609cm x 12mm / Wt: 1, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/01/25

Time of Operation

09:45 am

Location of Operation

#### 10 Tuas South St 5

CDI Energy Products Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Crowbar

Hydraulic/hand jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

# (1)

- Deploy lorry loader at designated unloading area

- Rigger to insert sling to crane hook

- Secured sling to rigging point of plate

- Hoist up plate from lorry chassis

- Swing crane and lower down plate to near ground level

- Insert pallet truck under plate base

- Transport plate into production area at ground floor

- Position plate at the designated location

(2)

- To jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from existing location

- Transport machine to new location within production area

- Position machine at the new designated location

- Jack up one side of machine, remove rollers and insert woodblock under machine bed

- Jack up another side of machine, remove rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175