# METHOD OF STATEMENT

Date: 03rd July 2024

Description of work

To unload 8 pcs metal plate from lorry chassis, shift and position into factory premise at ground floor

Spec

Metal plate

Dim: 400 x 210 x 4 cm

Wt: 1, 300 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/07/24

Time of Operation

13:00 pm

Location of Operation

#### 51 Marsiling Rd

Beyonics International Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to attached sling to crane hook

- Secured sling to rigging point of load

- Hoist up from lorry chassis

- Swing crane and lower down plate to the ground

- Insert rollers/pallet truck under plate

- Shift and transport into factory premise at ground floor

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175