# METHOD OF STATEMENT

Date: 26th Jan 2023

Description of work

To hoist 1 unit machine from lorry chassis to the ground, insert pallet truck to machine bed, shift and position into production at ground floor

Machine Spec

Ultrasonic Cleaner HC-48002F

Dim: 245 x 181 x 175 cm

Wt: 600 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/01/23

Time of Operation

09:45 am

Location of Operation

#### 12 Tuas Loop

Flowserve Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case

- Insert sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing crane away from base and lower down machine to the ground

- Insert pallet truck under machine bed

- Transport machine into production at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175