# METHOD OF STATEMENT

Date: 05th Apr 2024

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, insert pallet truck under machine bed, shift and position into workshop at level 1

Machine Spec

1 unit machine

Dim: 115 x 85 x 175 cm

Wt: 860 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/04/24

Time of Operation

10:00 am

Location of Operation

#### 500 Dover Rd, Singapore Polytechnic

#### Blk W515 level 1

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis to the ground

- Uncrate machine case

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Transport machine into workshop at level 1

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175