# METHOD OF STATEMENT

Date: 20th Mar 2023

Description of work

To hoist down 1 unit machine and accessory from lorry chassis to the ground, insert pallet truck to machine bed, shift and position into factory premise at ground floor

Machine Spec

2 cases of 1 unit ED30Z-50Z RDM

Case 1: 145 x 145 x 224 cm

Wt: 1, 350 kg

Case 2: 115 x 88 x 224 cm

Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27/03/23

Time of Operation

Afternoon schedule

Location of Operation

#### 61 Seletar Aerospace View

Pratt & Whitney Services Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case

- Insert sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing crane away from base and lower down machine to near ground level

- Insert pallet truck under machine bed

- Transport machine into factory premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175