# METHOD OF STATEMENT

Date: 08th Nov 2019

Description of work

To reverse low-bed to loading bay, jack up machine and insert rollers, shift out and position into tenant production area at ground floor

Machine Spec

VT650R MC machine

Dim: 3400 x 2400 x 3650 mm

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

09/11/2019   -   1pm

Duration of Operation

1pm – 5pm

Location of Operation

#### Blk 5000 A.M.K Ave 5 #01-09

Eratech Pte Ltd

Equipment Use

1 x low-bed

Heavy duty rollers

1 set hand jack

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# JOB SCOPE

- To reverse low-bed to loading bay directly in front of tenant louvre door

- Lift up machine using hand jack

- Insert rollers under machine base

- Shift out machine low-bed into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175