# METHOD OF STATEMENT

Date: 19th July 2019

Description of work

To unload and hoist down 3 cases of 2 unit machine from lorry chassis, uncrate, insert rollers to machine base, shift and position into production area

Machine Spec

1 unit Vesta 660

Dim: 380 x 220 x 255 cm / Wt: 4, 275 kg

1 unit Cutex 160A

Dim: 3780 x 1980 x 2160 mm / Wt: 4, 050 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/07/2019

Time of Operation

09:45 am

Location of Operation

#### 7 Soon Lee St #03-36/37

Winco Engineering Pte Ltd

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

1 set wire sling

1 set shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry crane to 3rd floor via ramp up

- Hoist down cases from lorry chassis to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to near door entrance

- Insert rollers/pallet truck under machine base

- Transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175