# METHOD OF STATEMENT

Date: 17th Oct 2019

Description of work

To hoist down and unload 3 packages of 1 unit machine from 1 x 40ft FR container, uncrate, insert rollers to machine base, shift and position into production area

Machine Spec

UG 800 CNC

Dim 1: 548 x 288 x 362 cm

Wt: 14, 000 kg

Dim 2: 303 x 108 x 188 cm

Wt: 1, 500 kg

Dim 3: 341 x 220 x 210 cm

Wt: 1, 900 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04-05/11/2019

Time of Operation

10:00 am

Location of Operation

#### Blk 16B Tuas Ave 1 #02-40/42, JTC Space @ Tuas

Rohag

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry crane to 2nd floor via ramp up

- Lorry crane to hoist down cases from container to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to near door entrance

- Insert rollers under machine base

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175