# METHOD OF STATEMENT

Date: 12th Apr 2024

Description of work

1) To hoist down 1 unit machine from lorry chassis to the ground, shift and position into factory premise at ground

2) To shift out 1 unit machine to loading area, hoist up onto lorry chassis and deliver to another location

Machine Spec

Move in

1 unit 5HP Milling

Move out

1 unit used Milling machine

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

16/04/24

Time of Operation

Afternoon schedule

Location of Operation

#### 15 Woodlands Terrace

Techcom Technology

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secured to machine rigging point

- Hoist up machine

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift into factory premise

- Position machine at the designated location

- Shift out used machine to loading bay

- Hoist up machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175