# METHOD OF STATEMENT

Date: 27th May 2024

Description of work

1) To unload and hoist down total 1 unit machine from lorry chassis, shift and position into production at ground floor

2) To shift out 1 unit used machine, hoist up onto lorry chassis and deliver to another location

Machine Spec

Move in

1 unit new Smart X-50 Digester Final 3D

Dim: 90 x 190 x 160 cm

Wt: 550 kg

Move out

1 unit used Smart X-50 Digester Final 3D

Dim: 90 x 190 x 160 cm

Wt: 550 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

12:00 am

Location of Operation

629 Lorong 4 Toa Payoh

ST Micro-Electronics Asia Pacific

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine from lorry chassis to the ground

- Release all sling from crane hook

- Insert pallet truck under machine bed

- Shift and transport machine into production at ground floor

- Position machine at the designated location

- Shift out 1 unit used machine to loading area

- Hoist up machine and load onto lorry chassis

- Released all sling from crane hook

- Secured machine with strapping belt

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175