# METHOD OF STATEMENT

Reference No: EW/Q/1072/23

Date: 29th Aug 2023

Description of work

To unload machine & accessories from customer arranged container, uncrate, shift and position into production area at ground floor

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

08/09/23

Time of Operation

09:45 am

Location of Operation

#### 12 Tuas Loop

Flowserve Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Hand jack

Woodblock

Wire/webbing sling

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry back to back with container

- Crane to pull out machine case from container onto lorry chassis

- Hoist down case to the ground

- Uncrate case and put aside

- Unscrew all nuts

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175