# METHOD OF STATEMENT

Date: 11th Apr 2021

Description of work

To unload 11 packages of 1 set machine from trailer/lorry chassis, uncrate, shift and position into production area at ground floor

Machine Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

Estimate: 18/04/22

Time of Operation

09:45 am

Location of Operation

#### 16 Tuas Ave 7

ST Kinetics Singapore

Equipment Use

633 lorry loader

1 x forklift

1-2 x trailer

Rollers / pallet truck

Hand jack / crowbar

Wire / webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Reverse trailer to near crane

- Hoist down case from trailer chassis to the ground

- Uncrate case and put aside

- Attached required lifting gears to crane hook and secured to rigging point of load to be lifted

- Lorry loader to hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175